| 1 | APPROVAL |  |  |  |
| --- | --- | --- | --- | --- |
| FUNCTION |  | NAME | DESIGNATION & DEPARTMENT | SIGNATURE & DATE |
| PREPARED BY |  | PRAJKTA LOLARE | JUNIOR EXECUTIVE QUALITY ASSURANCE DEPARTMENT | 22/12/2025 |
| REVIEWED BY |  | RAGHAVENDRA KATTI | MANAGER OPERATIONS | 23/12/2025 |
| APPROVED BY |  | Dr. VIKAS GHATTARGI | MANAGER QUALITY ASSURANCE DEPARTMENT | 23/12/2025 |
| 2 | REVISION HISTORY |  |  |  |
| REV. NO. |  | DESCRIPTION OF CHANGE |  |  |
| 00 |  | New SOP Preparation, Refer CCN/STR/251 |  |  |
| 01 |  |  |  |  |
| 02 |  |  |  |  |
| 03 |  |  |  |  |
| 04 |  |  |  |  |
| 05 |  |  |  |  |

| 3 | OBJECTIVE |  |  |  |
| --- | --- | --- | --- | --- |
| To lay down the procedure for functioning of stores in the Manufacturing facility of Achira Labs Pvt. Ltd. |  |  |  |  |
| 4 | SCOPE |  |  |  |
| This SOP is applicable for all raw & packing materials received at the Manufacturing facility of Achira Labs Pvt. Ltd. |  |  |  |  |
| 5 | RESPONSIBILITY & AUTHORITY |  |  |  |
| ROLE |  |  | RESPONSIBILITY |  |
| Head |  |  | Accountable for effective implementation of the procedure |  |
| Security Personnel |  |  | Responsible for informing stores regarding receipt of raw materials and collecting the consignment documents. |  |
| Stores Assistant, officers & executive |  |  | Responsible for verification of the documents & physical condition of the material in the vehicle. |  |
| 6 | REFERENCE AND RELATIONSHIP |  |  |  |
| S. No. |  | DOCUMENT No. |  | RELATIONSHIP |
| 1 |  | ISO 13485:2016, Clause 7.4.1 |  | Purchasing process |
| 2 |  | ISO 13485:2016, Clause 7.4.3 |  | Verification of purchased product |
| 3 |  | ISO 2859 |  | Sampling plans for attribute features |

| 7 | DEFINITIONS |  |  |  |
| --- | --- | --- | --- | --- |
| NA |  |  |  |  |
| 8 | ABBREVIATIONS |  |  |  |
| SHORT FORM |  | FULL FORM | SHORT FORM | FULL FORM |
| SOP |  | Standard Operating Procedure | STR | Stores |
| PO |  | Purchase Order | QA | Quality assurance |
| COA |  | Certificate of analysis | PR | Purchase requisition |
| MFG |  | Manufacturing | QC | Quality control |
| TMP |  | Template | FMT | Format |

| 9 |  |  | PROCEDURES |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Health and Safety - Ensure proper use and protect the health and safety during handling of biological material. - Avoid direct contact, inhalation and ingestion. - Use disinfectant on hands and wear hand gloves to avoid direct contact. - Wash the hands thoroughly after handling. - Material Receipt - On arrival of materials at the gate, security officer shall collect the documents received along with the material and handover the documents to Stores department. - Then stores In-charge shall verify the consignment against valid PO and related documents of the material received. - Check the physical condition & quantity of the material against DC / Invoice / Goods consignment Note / Docket. - In case the material does not meet the requirement as per the order placed, immediately the matter should be brought to the notice of the respective Heads. - Allow the material to unload on the clean pallet and transfer into the de-dusting area. - Detail entry to be done in the Material Inward Register(TMP/STR/001). - In case of damage packs, transfer into the new corrugated boxes. Remove the supplier label from the damage boxes and paste on the repacked corrugated box/ plastic crate. - Carry out de-dusting and Physical verification of the incoming material (For manufacturing process only) and record the detail in the respective template (TMP/STR/002). - Dedusting to be performed first by applying vacuum cleaning on the outer surface of the packages received. If necessary, apply dry cloth mopping of received material. - Physical Verification for Biological Raw Material should be 100%. - Physical Verification for Packing material as mentioned below as well as referring ISO 2859. - For Carton / Leaflet / Label/ Aluminium Foil Pouch. No. of Boxes received; Physical Verification of Boxes; Physical Verification by counts. 2 to 8 boxes; 2 boxes; One pack from each Box. 9 to 15 boxes; 3 boxes. 16 to 25 boxes; 5 boxes. 26 to 50 boxes; 8 boxes. For packs, more than above mentioned to refer in ISO 2859.. (B)For Plastic cassettes/cartridge/ cartridge parts follow above mentioned sample plan for packs and Physical Verification by counts/weight to be done by following formula- Wt of 25 nos X Net Wt of Packet 25 … (C) In case of Shipper physically count the Number of bundles multiply by shipper in each bundle. (D) In case of PVC roll, physically verify the quantity by weight. - In case of abnormal observation while verifying the quantity of received material, then keep it separately and inform to the person in charge. - Preparation of GRN - Respective entries to be done in template (TMP/STR/003) with respect batch No. to the received material. - Labelling - Print out the required quantity of Quarantine labels with all detail mentioned on each label. - Affix the labels on all containers/boxes/bundles. In case in the roll form then pasted inside the core of the roll. - Transfer the materials to the dedicated quarantine area / under testing area. - QC will affix “Under Testing” yellow colour label on the quarantine label of all containers(refer . - After testing QC, will paste the green colour approved label on all each container. - In case the material stands rejected, then QC will paste the red colour rejected label. - For all above labeling refer SOP/QAD/012. APPROVED; TMP/QAD/019. Material Name:; Date:. GRN No.:; Prod./Cat. Code:. Batch/Lot No.:; Item Code:. Release Qty.:; AR No.:. Retest Date:; Exp. Date:. Released By (Sign. /Date):. - Storage of Material - Approved material shifted to Approved area. - Rejected material to shift in separate isolated area and the information to be given to the QA & Purchase department for necessary action & recommendation. - Biological and other critical raw material should be stored accordingly to the storage conditions specified in the COA. - Photosensitive material to be stored in amber or black colored packaging.eg. Gold Chloride, Silver Nitrate etc. - Material inventory is taken in the Stock register. - Dispensing of Material - The requisition slip should receive from manufacturing & packing department duly signed by authorized person. - FEFO (First Expiry First Out) system to be followed for issuing the raw material. - FIFO (First In First Out) system to be followed issuing the packing material. - Balance verification to be done as per SOP. - After dispensing, loose quantity of the material should be closed tightly and shifts it into the original container. Consumption/ Stock log to be updated – refer (TMP/STR/004). - Expired Material - Expired raw materials should be informed to QA department for necessary action. - Keep track on expiry date of materials. - Packing material without Expiry Date to be retested after three years from date of receipt. - Indenting of Material - Indent (Purchase requisition) is raised by the stores / MFG department as per the manufacturing planning. - Material Indent (PR)is prepared by Store In-charge, approved by MFG head and authorized by Plant Head. - Interdepartmental transfer note - The TMP/STR/011 will control the interdepartmental transfer of materials, including raw materials, packing materials, biomaterials, engineering materials, and capital items. - Document Numbering System both D&D AND MFG S. No.; Groups; Inward Entry No (Register); GRN No. 1; Raw Material,Packing Material,Biomaterial,Engineering material; IN/RM/00N; GR/RM/00N. 2; D&D Material; IN/DD/00N; GR/DD/00N. 3; Capital Goods (e.g., Refrigerators, Oven, Coating M/c. Computers, Racks, S.S. Furniture’s, Plastic Bins, Weighing balance, Xerox M/c & etc.; IN/CG/00N; GR/CG/00N. 4; Services (e.g., Pest Control, AHU, Consultancy, Calibration & etc.); IN/SE/00N; GR/SE/00N. 5; Miscellaneous Material (e.g., Office stationery, housekeeping material, grocery, provisional & etc.); IN/CC/00N; GR/CC/00N. |
| 10 |  |  | RECORDS |  |  |  |
| RECORD NO. |  |  |  | DESCRIPTION |  | STORAGE & RETENTION PERIOD |
| TMP/STR/001 |  |  |  | MATERIAL INWARD REGISTER |  | QAD, 3 Years |
| TMP/STR/002 |  |  |  | DE-DUSTING AND PHYSICAL VERIFICATION |  |  |
| TMP/STR/003 |  |  |  | PREPARATION OF GRN |  |  |
| TMP/STR/004 |  |  |  | CONSUMPTION LOG |  |  |
| TMP/STR/011 |  |  |  | INTERDEPARTMENTAL TRANSFER NOTE |  |  |
| 11 | MONITORING & MEASUREMENTS |  |  |  |  |  |
| SOP is to be reviewed at least every two years. This shall be tracked using the Master Document List by entering a date two years from the effective date. |  |  |  |  |  |  |
| 12 | RISK MANAGEMENT |  |  |  |  |  |
| S. No. |  | HAZARD |  |  | CONTROL MEASURES |  |
| 1 |  | Breakage of material during Pickup or Movement of material. |  |  | Hooks are not used for pickup or movement of the material containers/bags. |  |
| 2 |  | During Inspection of raw materials, in vehicle if containers/drums/ bags are found to be tampered. |  |  | Personnel verifies the materials as per the checklist |  |